Openbmc

Flash Ubuntu and QEMU Installation:

- Flash ubuntu 24.04.2 LTS

- Install pre requisites as below:

- sudo apt install -y git build-essential libglib2.0-dev libfdt-dev libpixman-1-dev zlib1g-dev ninja-build python3

- sudo apt install git gcc g++ make file wget gawk diffstat bzip2 cpio chrpath zstd lz4 bzip2 libslirp0

- Install QEMU using following command:

wget https://jenkins.openbmc.org/job/latest-qemu-x86/lastSuccessfulBuild/artifact/qemu/build/qemu-system-arm

- Give the command chmod u+x qemu-system-arm for permissions.

- Check the version by using command:

./qemu-system-arm –version

Clone openbmc and build the image for AST2600 using QEMU:

Steps:

- git clone https://github.com/openbmc/openbmc.git

- cd openbmc

- .  setup evb-ast2600

- bitbake obmc-phosphor-image

if any particular error occurs then clean and compile

Example: bitbake -c cleansstate srvcfg-manager

bitbake -c compile srvcfg-manager -f -v

then again give bitbake obmc-phosphor-image

If any permission issue, try the command:

echo 0 | sudo tee /proc/sys/kernel/apparmor_restrict_unprivileged_userns

- Once build is successful, go to the below path :

cd /home/openbmc/<your_folder>/openbmc/build/evb-ast2600/tmp/deploy/images/evb-ast2600/

- Execute the command:

./qemu-system-arm   -m  512  -M  ast2600-evb   -nographic    -drive file=obmc-phosphor-image-evb-ast2600.static.mtd,format=raw,if=mtd    -net nic     -net user,hostfwd=tcp::2222-:22,hostfwd=tcp::2443-:443,hostfwd=udp::2624-:623,hostname=qemu

Here command defines:

| Option | Description |
| --- | --- |
| qemu-system-arm | Launches the QEMU ARM emulator, which simulates an ARM-based system. |
| -m 512 | Allocates 512 MB of RAM to the virtual machine. |
| -M ast2600-evb | Uses the AST2600 Evaluation Board as the virtual machine platform (specific to ASPEED BMC SoCs). |
| -nographic | Disables graphical output and redirects the serial console to the terminal. Useful for headless systems or servers. |
| -drive file=obmc-phosphor-image-evb-ast2600.static.mtd,format=raw,if=mtd | Loads the MTD (flash) image created by OpenBMC (.mtd format) as a raw flash drive attached using MTD interface. |
| -net nic | Adds a default virtual Network Interface Card (NIC) to the virtual machine. |
| -net user,... | Enables user-mode networking (no special privileges required) and allows port forwarding from the host to the guest. |
| hostfwd=tcp::2223-:22 | Forwards host's port 2223 to guest's port 22 (SSH). So you can SSH into the guest using: ssh root@localhost -p 2223 |
| hostfwd=tcp::8444-:443 | Forwards host's port 8444 to guest's port 443 (HTTPS). Useful if the BMC serves a web UI. |
| hostfwd=udp::2624-:623 | Forwards host's UDP port 2624 to guest's UDP port 623, typically used for IPMI over LAN. |
| hostname=qemu | Sets the hostname inside the guest VM to qemu. |

Note: If any error related to qemu, then install the qemu again mentioned in QEMU installation steps.

- Once the Image get booted successfully, then give the credentials:

Username: root

Password: 0penBmc(not capital O, its zero)

- Login Successful and the image running successfully.

BOOT FLOW:

- Go to the path /tmp/deploy/images/evb-ast2600/

and find the images image-bmc, u-boot.bin, u-boot-spl.bin, fitImage and flash-image or flash-ast2600.bin

u-boot-spl.bin:

Purpose: Secondary Program Loader(SPL)

- Role: This is the first stage bootloader that runs immediately after the ROM boot code.

- Function: Initializes minimal hardware (DRAM, clocks, etc) then loads the full U-Boot(u-boot.bin)

- Size: small(usually <200KB)

- Flash location: Starts at offset 0x0 in SPI NOR.

u-boot.bin:

Purpose: Main Bootloader (U-boot proper)

- Role: This is the second stage bootloader, located by u-boot-spl

- Function: Provide a shell(console), initializes more subsystems, loads kernel image(eg., fitImage), boots OS.

- Flash Location: Typically after SPL(e.g., offset 0x80000 in SPI NOR)

- Features:

- Environment variables

- TFTP/NFS support

- Flash and memory read/write

fitImage:

Purpose: Flattened Image Tree(kernel+DTB+initramfs)

Contains: Linux kernel(zImage)

DTB(device tree blob)

Initramfs(optional embedded rootfs)

Format: fitImage is a Flattened Image tree(FIT) format with digital signature support.

Image-bmc:

Purpose: BMC Root Filesystem

Role: This is the main root filesystem image used by OpenBMC

Contents: System binaries, daemons, networking tools, etc

Format: ext4 image

Usage: Mounted as / root filesystem by the kernel

flash-ast2600.bin/flash-image:

Purpose: Monolithic Flash Image(Full SPI nor content)

Role: This is a complete flashable image combining:

SPL, U-Boot, kernel, RootFs

Usecase: Write directly to SPI NOR Flash via flashcp, dd, UART recovery.

BOOT MODE:

SPI NOR Boot Mode.

- Need to physically set jumpers or DIP switches on EVB-AST2600 board.

- Locate the BOOT_SEL jumpers or switches on the board:

- They are usually labeled as BOOT_SEL0, BOOT_SEL1 or BOOT_MODE[0:1].

- Set both BOOT_SEL0 and BOOT_SEL1 to 0 (Low):

This means short the pins to GND or place jumpers/switches to LOW position.

Flashing the image into board:

Flash using JTAG (e.g., with ASPEED AST2600 SDK tools)

- You can use tools like Aspeed SDK's ast-flash utility.

- This requires a JTAG adapter (e.g., USB-Blaster or FTDI-based).

After flashing:

- Power off the board.

- Make sure BOOT_SEL = 00.

- Power on the board.

Hardware:

- EVB based on ASPEED AST2600

- FTDI-based JTAG debugger (e.g., FT2232H)

- JTAG-to-board connection (usually through a 2x10 20-pin JTAG header or test points)

- UART-to-USB for serial logs (optional, but strongly recommended)

Software on Host (Linux):

- ast-flash tool (from ASPEED SDK or OpenBMC utilities)

- FTDI drivers installed

- Optionally: libftdi1-dev, libftdi1, libusb-1.0-0-dev

- flash-evb-ast2600 image available on host PC

Steps:

Step-1: Get the ast-flash tool:

Build ast-flash manually:

git clone https://github.com/AspeedTech-BMC/aspeed-bmc-utils.git

cd aspeed-bmc-utils/ast-flash

make

NOTE: This will generate the ast-flash binary.

Step-2: Connect JTAG to Board

- Connect FTDI JTAG pins to the EVB as per your board’s JTAG pinout. Usually:

    - TCK → TCK

    - TDI → TDI

    - TDO → TDO

    - TMS → TMS

    - GND → GND

    - TRST, SRST (optional but recommended)

Note: Make sure to power the board before flashing.

Step 3: Flash the image using ast-flash

Command: sudo ./ast-flash -f flash-evb-ast2600

By default, this tries to detect the JTAG interface and writes the full SPI image.

Step 4: Power-cycle and Boot

- Disconnect JTAG.

- Ensure:

    - BOOT_SEL[1:0] = 00 (SPI NOR)

    - Board is powered

- Open a serial terminal to observe the booting.

Out-of-band Management VS In-Band Management:

| Feature | Out-of-Band Management | In-Band Management |
| --- | --- | --- |
| BMC Access Path | Accessed via dedicated network interface (BMC IP) | Accessed through the host OS |
| Host OS Dependency | Independent of the host OS (works if OS is off/crashed) | Requires the host OS to be running |
| Management Interface | BMC has its own OS (like OpenBMC) and runs independently | No separate OS; uses host system drivers/interfaces |
| Network Requirement | Requires dedicated IP for BMC | Uses the same network as the host system |
| Tools/Access Methods | IPMI over LAN, Redfish, SSH to BMC, Web UI | ipmitool, busctl, Redfish via host, kernel drivers |
| Power Control (On/Off/Reset) | Yes | Not possible without host OS |
| Sensor Monitoring | Yes | Yes (as long as OS is running) |
| Firmware Update Capability | Yes | Yes (via host-initiated tools) |
| Common Use Cases | Recovery, remote management, diagnostics, power control | Monitoring, logging, BMC updates via software |
| Works When Server is Off? | Yes | No |
| Security Scope | Separate management channel, often secured separately | Relies on host OS security policies |

Summary:

- Out-of-Band is external control via the BMC, even when the main server is off or broken.

- In-Band is internal control through the running operating system.

IPMI vs Redfish:

| Feature | Redfish | IPMI |
| --- | --- | --- |
| Protocol Type | RESTful API (Web-based, HTTP/HTTPS) | Command/response protocol (binary, UDP or driver-based) |
| Data Format | Human-readable JSON | Binary |
| Ease of Use | Easy (web tools, scripts, browsers) | Harder (special tools like ipmitool needed) |
| Security | Strong (HTTPS, TLS, Role-based access) | Weaker (older versions use MD5 or no encryption) |
| Modern Support | Designed for modern systems & automation | Legacy system, older architecture |
| Extensibility | Schema-driven, easily extended | Fixed command set, hard to extend |
| Tool Support | curl, redfishtool, browser, Python SDK | ipmitool, FreeIPMI, IPMIView |
| Performance | Slower (uses HTTP) but scalable & reliable | Faster (binary-based), but limited functionality |
| Visualization | Can be used with dashboards or web UIs | Mostly CLI-based |
| Firmware Updates | Fully supported, with detailed progress | Supported, but less structured |
| Adoption Trend | Increasing (used in OpenBMC, Dell iDRAC, etc.) | Declining (but still widely supported) |

Summary:

- Redfish is modern, secure, easy to integrate with web/cloud tools, and more future-proof.

- IPMI is older, less secure, but still commonly used due to compatibility with existing infrastructure.

PLDM(Platform level Data Model):

PLDM allows the BMC to manage, control, and monitor devices or firmware using a lightweight, structured protocol — instead of heavier protocols like IPMI or Redfish.

Inside the system, when the BMC communicates with other components (like BIOS, CPU, PCIe devices, sensors, etc.), it increasingly uses PLDM, rather than older protocols like IPMI.

- Internally = communication within the server, between:

    - BMC ↔ BIOS

    - BMC ↔ CPLD/FPGA

    - BMC ↔ Storage devices (e.g., NVMe)

    - BMC ↔ PCIe cards

    - BMC ↔ Power/thermal sensors

These communications are not exposed to users directly like Redfish/IPMI. They're happening within the system, often over buses like:

- MCTP over I²C

- MCTP over PCIe

PLDM  ----- passing data between the BMC and internal hardware.

- PLDM talks to hardware, BMC collects data

Example:

- A PLDM sensor command is sent to a PCIe device or a fan controller.

- The device replies with temperature, voltage, status, etc.

- BMC stores this data internally.

- IPMI or Redfish will display the data to the user.

MCTP (Management Component Transport Protocol)

MCTP is a low-level transport protocol used in modern server platforms to allow management communication between hardware components, especially in platforms using OpenBMC.

For Example:

The BMC wants to monitor and control a PCIe-based network card using the PLDM protocol over the MCTP transport layer.

System Components:

- BMC (Baseboard Management Controller) running OpenBMC

- PCIe Network Interface Card (NIC) that supports PLDM

- Communication via MCTP over PCIe VDM (Vendor Defined Messaging)

Step-by-Step Flow:

- BMC Interaction with PCIe:

- The BMC wants to read or control hardware, such as a PCIe device (e.g., network card, storage controller).

- Since the PCIe device supports PLDM (Platform Level Data Model), the BMC uses PLDM to communicate with the device for monitoring or control purposes.

- PLDM over MCTP:

- PLDM defines a set of commands and data formats that the BMC uses to interact with devices. However, PLDM itself doesn’t handle transport.

- The MCTP (Management Component Transport Protocol) is the transport layer that carries PLDM commands between the BMC and PCIe devices (like sensors, FRU data, etc.).

- The BMC sends PLDM requests (e.g., "Get Sensor Reading") using MCTP. The MCTP protocol is responsible for routing these commands to the appropriate PCIe device, which then processes the request and returns the data (e.g., sensor readings, status).

- BMC Fetches Data:

- The PCIe device processes the PLDM request and responds to the BMC with the requested data (e.g., temperature, voltage levels, etc.).

- The BMC may also send PLDM control commands, such as resetting a device, configuring its settings, or updating its firmware.

- Displaying Data to Users:

- Once the BMC has collected the data from the PCIe device via PLDM over MCTP, it needs to present this information to the user.

- IPMI or Redfish are used for this purpose, as they are the standard protocols for managing and querying BMC data.

- IPMI provides a set of commands for monitoring and controlling hardware, including accessing sensor data or performing control operations.

- Redfish provides a modern, RESTful interface that allows users to query BMC data in a structured manner (typically JSON format), providing a more flexible, scalable management solution compared to IPMI.

- User Interaction:

- The user can query the BMC (using Redfish or IPMI) for system information, such as the temperature of the PCIe card, its status, or other relevant hardware details.

- If the user interacts through Redfish (via a web UI or REST API), the BMC responds with data retrieved from the PCIe device, which may have been fetched using PLDM over MCTP.

Systemd:

systemd is a system and service manager for Linux. It's responsible for starting up the system, managing services and handling shutdowns.

Think of systemd as the "conductor" of a Linux system:

- It decides which processes start, in what order, and under what conditions.

- It replaces older init systems like SysVinit or init.d.

- systemd reads all enabled units during boot (or when a target is triggered)

It checks unit files for:

- Requires=, Wants= → services to start along with the current one

- Before=, After= → services to start in correct order

Then it executes services in the proper order — without needing any manual commands

D-BUS:

D-Bus (Desktop Bus) is an Inter-Process Communication (IPC) system.
In simple terms, it’s a message bus — a way for different programs/processes to talk to each other on a Linux system.

In OpenBMC, almost everything runs as a D-Bus service. It’s the core communication mechanism between:

- Power control services

- Sensors

- Fan and thermal management

- Inventory information

- Host state machines

- External interfaces (IPMI, Redfish)

NOTE:

If you remove D-Bus from OpenBMC, most services would stop working.

Step-by-step example scaneario:

Step 1: BMC Powers ON

- The OpenBMC firmware starts running.

- It loads the Linux kernel.

Step 2: systemd Starts

- systemd is the first process started after the kernel.

- It reads all the service files (unit files like .service files).

- Then, systemd starts services like:

  - D-Bus

  - ObjectMapper

  - Sensor services

systemd's job is to start and monitor services. It doesn't store or read data.

Step 3: Services Register on D-Bus

- Once started, each service connects to D-Bus and says:

“Hey, I can provide sensor data!”

- For example, the temperature sensor service says:

“I’m the owner of /xyz/openbmc_project/sensors/temperature/cpu0 and I can give you the temperature.”

Step 4: Sensor Service Reads Hardware

- The sensor service talks to the hardware (via I2C or another interface).

- It reads the temperature value from a chip or board.

- It stores the value in memory and makes it available on D-Bus.

Step 5: You (the user) want to read the sensor

You can use:

- busctl (Linux command to talk to D-Bus)

- ipmitool (to use IPMI interface)

- curl or browser for Redfish API

In Summary:

[BMC powers on]

↓

[systemd starts]

↓

[systemd starts Sensor Service + D-Bus]

↓

[Sensor Service reads data from hardware]

## ↓

[Sensor Service makes data available on D-Bus]

↓

[User uses busctl/ipmitool/Redfish to read from D-Bus]

BMC Hardware architecture and Block diagram:

1.Motherboard Side (Left Side):

- Network (LAN) Controller:

- BMC communicates externally using the Network (LAN) Controller.

- The RJ45 connector is the physical port through which you connect your laptop or server network to the BMC.

- This port allows access to BMC services such as:

- Web UI (if supported)

- SSH

- IPMI over LAN (ipmitool -I lanplus)

- Redfish API

- LAN Connector:

- The physical RJ45 port for network connectivity to the BMC or host.

- Serial Port Sharing / Motherboard Serial Controller:

The serial connector in the diagram can refer to multiple serial paths:

1.Serial Port Sharing (from motherboard to BMC):

Serial Port Sharing means the BMC is connected to the host's serial console, and it    can:

- Monitor the host serial output (like BIOS logs, kernel boot logs).

- Relay this output over the network, so remote users can see or interact with the host console using:

- IPMI Serial-over-LAN (ipmitool -I lanplus -H <ip> -U <user> -P <pass> sol activate)

- Web console or SSH (depending on BMC features)

How it works step-by-step:

- The host (CPU) boots and outputs logs to its serial port.

- This serial output is routed to the BMC via "Serial Port Sharing".

- The BMC captures that serial data and exposes it over the network.

- You (admin/developer) connect to the BMC (via SSH or IPMI) and run Serial Over LAN (SOL) to:

  - See host boot logs

  - Interact with the BIOS

  - Run OS-level commands if host is running Linux

2.Serial Connector (top left corner):

- This may be used for direct serial access to the BMC itself, depending on design.

- Often used by developers or field engineers for low-level debugging or flashing.

- Baseboard Management Controller (BMC) (Center):

This is the brain of the platform management. It operates independently from the host CPU and OS.

- Serial Controller (within BMC):

- Handles communication over serial lines to other components.

- PCI Management Bus / IPMB (Intelligent Platform Management Bus:

  The PCI Management Bus shown is more of a connection bridge from host to BMC for side-band management tasks.

  But sensors like temperature, voltage, fans, etc. are typically connected to the BMC through I²C/SMBus, not PCI.

  PCI is used for host-side management chips or optional BMC-accessible resources.

- Private Management Busses:

- Busses like I²C/SMBus that connect to sensors, EEPROMs, etc.

- System Interface (IPMI messages):

The System Interface between the BMC and the host CPU allows the host to send IPMI commands to the BMC (and vice versa). This communication typically happens over KCS (Keyboard Controller Style), BT (Block Transfer), or SMBus.

- This is how the host OS or BIOS can:

- Log events (e.g., system overheating)

- Read BMC logs (SEL, sensors)

- Configure watchdogs

- Request BMC actions (e.g., reboot, power off)

- Non-Volatile Storage:

Used by BMC to store:

- SEL (System Event Log): Log of hardware events (e.g., overtemperature).

- SDR (Sensor Data Records): Metadata about each sensor.

- FRU (Field Replaceable Unit) info: Information about components like fans, power supplies, etc.

- Sensors and Control Circuitry:

These provide real-world data such as:

- Voltages

- Temperatures

- Fan speeds

- Power status

The BMC reads this data over private buses (like I²C) and shares it via IPMI or D-Bus (in OpenBMC).

- FRU EEPROMs:

EEPROMs store the FRU data (e.g., vendor, part number, serial number) of hardware units like:

- Temperature sensors

- Power supplies

- Fans

- Chassis Management / Satellite Controllers (Right Side):

This block is responsible for monitoring and controlling the physical chassis hardware, such as:

- Fans

- Temperature sensors

- Power supplies

- Chassis identification

- Redundant power handling

- Chassis-wide FRU (Field Replaceable Unit) data

How It Works:

- It gathers real-time data from chassis sensors (bottom-right area: "Chassis Sensors" like temperature, fans).

- It stores FRU data in EEPROMs (like component serial numbers, part numbers, vendor info).

- It communicates with the BMC to send this information, usually over I²C/SMBus or a private management bus.

So, the BMC collects health and identity info from the Chassis Management controller and makes it available over IPMI, Redfish, etc.

- Remote Management:

This allows remote users or management systems (e.g., from a laptop or data center tool) to access and manage the BMC over the network or serial.

It supports:

- Remote terminal access (via Serial or SSH)

- Web UI / Redfish API / IPMI over LAN

- Monitoring and controlling power, sensors, and logs

- Firmware updates remotely

Interfaces:

- LAN (RJ45): Connects to the BMC via the Network Controller — allows tools like ipmitool, Redfish, or web UIs to communicate.

- Serial Connector: Allows serial terminal access to the BMC shell (like UART debug).

- Modem (optional): For dial-up style legacy remote management.

- ICMB (Intelligent Chassis Management Bus): Optional legacy IPMI bus between different BMCs in multiple chassis (rare today).

- IPMI Message Flow:

Example: Reading CPU Temperature.

- BMC powers on → systemd starts → sensor service starts

- 2. Sensor service reads temperature from sensor via I²C

- 3. BMC stores value in memory / makes it available on D-Bus/IPMI

- 4. User sends command using `ipmitool sensor list`

- 5. IPMI message is handled by BMC and value is returned to user