Fronthaul.cpp

namespace FronthaulIf {

It Defines a C++ namespace called FronthaulIf. This namespace is used to have all the functions and variables declared in it.

Function configureCc

This function is  for configuring a component carrier. It takes Four parameters

CC, cfg, ulTiming, dlTiming.

An object of type `XorifUlTimingParameters` that likely represents uplink timing parameters.

dlTiming: An object of type `XorifDlTimingParameters` that likely represents downlink timing parameters.

std::cout<< "Entering configureCc\n";

It prints Entering configureCc.

It then makes several calls to the `RUN_CHECK_LOG_RETURN_RC` macros. These macros used for running a function, checking its return value, printing errors, and returning  results.

RUN_CHECK_LOG_RETURN_RC(xorif_et_cc_num_rbs, 0, cc, cfg.numRbs);

In xorif_set_cc_num_rbs is function  with three arguments 0, cc, and cfg. numRbs. And so on.

After these configuration calls, the function prints the values of some of the configuration parameters It prints std::cout:

cfg.numRbs

cfg.numerology.scs

cfg.numerology.cpType

These lines of code continue the configuration of a Component Carrier (CC) in a communication system using the `RUN_CHECK_LOG_RETURN_RC` macro. Let's break down each line:

The first RUN_CHECK_LOG_RETURN_RC call invokes the xorif_set_cc_dl_timing_parameters function  with several arguments.

0 This is likely a default or constant.

cc The Component Carrier number.

dlTiming.delayCompCp A parameter related to delay component of the downlink.

dlTiming.delayCompUp A parameter related to delay compensation for uplink timing.

dlTiming.advance advanceparameter for downlink timing.

This call configure the downlink timing parameters for the specified CC.

The second RUN_CHECK_LOG_RETURN_RC call invokes the xorif_set_cc_ul_timing_parameters function with following arguments:

0-default or constant

cc The Component Carrier number.

ulTiming.delayCompCp A parameter related to delay compensation for the uplink.

ulTiming.advance An advance parameter for uplink timing.

ulTiming.radioChDelay A parameter related to radio channel delay for uplink.

This call configure the uplink  timing parameters for the specified CC.

The third `RUN_CHECK_LOG_RETURN_RC` call invokes the `xorif_set_cc_dl_iq_compression` function  with  arguments:

0 -Likely a default or constant argument.

cc - The Component Carrier number.

cfg.compressionDl.bitWidth Bit width configuration for downlink IQ  data compression.

cfg.compressionDl.method used for downlink IQ data compression.

cfg.compressionDl.useStaticConfigByMplane related to the M-plane

This call appears to configure downlink IQ data compression settings for the specified CC.

After the third configuration call, two std::cout statements are used to print output. These lines print the Component Carrier number  and of the configuration parameters related to downlink IQ data compression:

cc The Component Carrier number.

cfg.compressionDl.bitWidth Bit width configuration for downlink IQ data.

cfg.compressionDl.method used for downlink IQ data compression.

Like that all the functions been called and setting values.

int initialize(const std::string & ifName)

This code defines initialize function to initialize respected parameters related to an O-RAN interface.

The function takes following parameter:

ifName

The function begins by making several calls to the RUN_CHECK_LOG_RETURN_RC macro, used to run functions, checking return values, and return values The respected  functions being called and their output depend on the the system and the O-RAN interface.

The first RUN_CHECK_LOG_RETURN_RC call invokes the `xorif_init` function with the following arguments:

0

nullptr A null pointer

This call  initializes the O-RAN interface.

The second RUN_CHECK_LOG_RETURN_RC call invokes the `xorif_reset_fhi` function with the argument 0. This call reset

The third RUN_CHECK_LOG_RETURN_RC call invokes the `xorif_set_fhi_protocol` function with following arguments:

0

xorif_transport_protocol PROTOCOL_ECPRI: An enumeration representing a transport protocol.

1

xorif_ip_mode

This call  set the transport protocol

The fourth `RUN_CHECK_LOG_RETURN_RC` call invokes the `xorif_set_ru_ports` function  with arguments representing port configurations.

The code is to read the MAC (Media Access Control) address of the interface from the `/sys/class/net/<ifName>/address` file, where `<ifName>` is the name of the interface passed as an argument to the function.

If the MAC address is successfully read from the file, it is converted from a string to an array of bytes using the `Utils::macStringToArray` function.

The MAC address is then set using the `xorif_set_fhi_src_mac_addr` function  with the following arguments:

0

The MAC address data from the file.

This call appears to set the source MAC address for the O-RAN interface.

If any errors occur during the MAC address reading or setting process, error message is printed using the Logger::critical function. The function returns -1 to indicate a failure.

If all the initialization steps are successful, the function returns 0 to indicating successful initialization.

int setEaxcMapping(const EaxcMappingConfig &cfg)

This function takes a single argument, cfg which is a reference to an object of type `EaxcMappingConfig`. The purpose of this function is to set EAXC (Ethernet over O-RAN Fronthaul) mapping configuration.

It calls the xorif_set_fhi_eaxc_id function  with the values cfg object, cfg.du_bits, cfg.bs_bits, cfg.cc_bits, and cfg.ru_bits.

The return value of this function is the return value of the xorif_set_fhi_eaxc_id function.

bool isOranByteSwapped(const xorif_iq_comp method)

This function takes one argument, method, which is of type xorif_iq_comp. It checks whether byte swapping is required for compression method.

Inside the function, there is a `switch` statement that validate the method argument.

If method is xorif_iq_comp::IQ_COMP_BLOCK_FP, the function returns false, indicating that byte swapping is not required for this compression method.

If method is xorif_iq_comp::IQ_COMP_NONE, the function returns true,  indicating that byte swapping is required for this compression method.

If method is one of the other cases , the function logs a critical error message using Logger::critical() and then returns false.

namespace FronthaulIf

This code defines a namespace called `FronthaulIf`, which contains the two functions mentioned above. Namespaces are used to group related functions, classes, or variables.

Values passing to the arguments

xorif_set_ru_ports :

cc =0

ru_bits =5

ss_bits=3

mask =24

user_val=0

prach_val =8

ssb_val =16.

xorif_set_fhi_protocol:

xorif_transport_protocol transport = 0

vlan =1

xorif_ip_mode = IP_MODE_RAW

xorif_set_cc_dl_sections_per_symbol:

cc =0

num_sect =91

num_ctrl =91

xorif_set_cc_ul_sections_per_symbol

cc =0

num_sect =91

num_ctrl =91

xorif_set_cc_sections_per_symbol_ssb

cc =0

num_sect =7

num_ctrl =7

xorif_set_cc_frames_per_symbol

cc=0

num_frames =9

xorif_set_cc_frames_per_symbol_ssb

cc=0

num_frames =1

xorif_configure_cc

cc=0