MRU Beamer Register Map  - BEAMv0.5.0

| Register | Bits | Type | Name | Bit Num | Description | Value |
| --- | --- | --- | --- | --- | --- | --- |
| 0x0 | [31:0] | RO | IP_VERSION | 32 | IP core version in the following X.Y.Z format <Version_Num[7:0], Revision_Num[7:0], Patch_Num[7:0]> | n/s |
| 0x1 | [0] | RW | TOTAL_KILL | 1 | A synchronous reset of total IP block. The high value corresponds to the reset state | 0 |
|  | [1] | RW | DL_KILL | 1 | A synchronous reset of DL Beamer module | 0 |
|  | [2] | RW | UL_KILL | 1 | A synchronous reset of UL Beamer module |  |
|  | [3] | RW | PRACH_KILL | 1 | A synchronous reset of PRACH Beamer module |  |
|  | [4] | RW | SRS_KILL | 1 | A synchronous reset of SRS Processor module |  |
|  | [5] | RWP | STAT_KILL | 1 | A synchronous reset of total statistic counters |  |
|  | [6] | RWP | DL_STAT_KILL | 1 | A synchronous reset of downlink statistic counters |  |
|  | [7] | RWP | UL_STAT_KILL | 1 | A synchronous reset of uplink modules statistic counters (UL, PRACH, SRS) |  |
| General Configuraion |  |  |  |  |  |  |
| 0x2 | [0] | RW | DL_BEAM_EN | 1 | Enable DL beamforming | 1 |
|  | [1] | RW | UL_BEAM_EN | 1 | Enable UL beamforming | 1 |
|  | [2] | RW | PRACH_BEAM_EN | 1 | Enable PRACH beamforming | 1 |
|  | [3] | RW | DL_WEIGHT_SRC | 1 | Source of DL Beam weights: 0 – Code Book only, 1 – mixed CB + DU provided | 0 |
|  | [4] | RW | UL_WEIGHT_SRC | 1 | Source of UL Beam weights: 0 – Code Book only, 1 – mixed CB + DU provided | 0 |
|  | [5] | RW | PRACH_WEIGHT_SRC | 1 | Source of PRACH Beam weights: 0 – Code Book only, 1 – mixed CB + DU provided | 0 |
|  | [15:8] | RW | DL_BEAM_ID_NUM | 8 | Number of Beams for DL Beamer. Supported range of BeamIDs is equal to [0: DL_BEAM_NUM-1] | 1 |
|  | [23:16] | RW | UL_BEAM_ID_NUM | 8 | Number of Beams for UL Beamer. Supported range of BeamIDs is equal to [0: UL_BEAM_NUM-1] | 1 |
|  | [31:24] | RW | PRACH_BEAM_ID_NUM | 8 | Number of Beams for DL beamer. Supported range of BeamIDs is equal to [0: PRACH_BEAM_NUM-1] | 1 |
| 0x3 | [1:0] | RW | DL_BEAM_EMUL_SCENARIO | 2 | DL Beamer emulation scenario: 0 – Round-robin of 8 input eAxC to 32 antennas 1 – four combinations of Pi/2 rotation per each of 8 eAxC to 32 antennas 2-3 – not supported yet |  |
|  | [3:2] | RW | UL_BEAM_EMUL_SCENARIO | 2 | UL Beamer emulation scenario: 0 – Round-robin of 8 input eAxC to 32 antennas 1 – four combinations of Pi/2 rotation per each of 8 eAxC to 32 antennas 2-3 – not supported yet |  |
|  | [5:4] | RW | PRACH_BEAM_EMUL_SCENARIO | 2 | PRACH Beamer emulation scenario: 0 – Round-robin of 8 input eAxC to 32 antennas 1 – four combinations of Pi/2 rotation per each of 8 eAxC to 32 antennas 2-3 – not supported yet |  |
| 0x4 | [0] | RW | DL_UDCOMPHDR_RMV | 1 | Remove udCompHdr in DL input data stream 0 – disable, 1 – enable | 1 |
|  | [1] | RW | DL_IQ_BYTE_SWAP | 1 | Byte swapping of each IQ-component of DL input data stream: 0 – disable, 1 - enable | 1 |
|  | [2] | RW | UL_UDCOMPHDR_ADD | 1 | Add udCompHdr in UL output data stream 0 – disable, 1 – enable | 1 |
|  | [3] | RW | UL_IQ_BYTE_SWAP | 1 | Byte swapping of each IQ-component of UL output data stream: 0 – disable, 1 – enable | 1 |
|  | [4] | RW | PRACH_UDCOMPHDR_ADD | 1 | Add udCompHdr in PRACH output data stream 0 – disable, 1 – enable | 1 |
|  | [5] | RW | PRACH_IQ_BYTE_SWAP | 1 | Byte swapping of each IQ-component of PRACH output data stream: 0 – disable, 1 – enable | 1 |
|  | [6] | RW | SRS_UDCOMPHDR_ADD | 1 | Add udCompHdr in SRS output data stream 0 – disable, 1 – enable | 1 |
|  | [7] | RW | SRS_IQ_BYTE_SWAP | 1 | Byte swapping of each IQ-component of SRS output data stream: 0 – disable, 1 – enable | 1 |
| Frame Configuration |  |  |  |  |  |  |
| 0x8 | [3:0] | RW | DL_SUBFRAME_NUM | 4 | Number of subframes per frame | 2 |
|  | [11:4] | RW | DL_SLOTS_NUM | 8 | Number of slots per frame | 20 |
|  | [15:12] | RW | DL_SYMB_NUM | 4 | Number of symbols per slot | 14 |
|  | [24:16] | RW | DL_RB_NUM | 9 | Number of resource blocks (RB) per symbol | 273 |
|  | [28:25] | RW | DL_SC_NUM | 4 | Number of subcarriers per RB | 12 |
| 0x9 | [12:0] | RW | DL_SYMB_FFT_SIZE | 13 | OFDM Symbol FFT size | 4096 |
| 0xA | [3:0] | RW | UL_SUBFRAME_NUM | 4 | Number of subframes per frame | 2 |
|  | [11:4] | RW | UL_SLOTS_NUM | 8 | Number of slots per frame | 20 |
|  | [15:12] | RW | UL_SYMB_NUM | 4 | Number of symbols per slot | 14 |
|  | [24:16] | RW | UL_RB_NUM | 9 | Number of resource blocks (RB) per symbol | 273 |
|  | [28:25] | RW | UL_SC_NUM | 4 | Number of subcarriers per RB | 12 |
| 0xB | [12:0] | RW | UL_SYMB_FFT_SIZE | 13 | OFDM Symbol FFT size | 4096 |
| Test Generator Configuration |  |  |  |  |  |  |
| 0x10 | [0] | RW | TEST_SIG_GEN_EN | 1 | Generating output test signal instead ORAN output | 0 |
|  | [1] | RW | TEST_SIG_SRC | 1 | Source of for test signal generating: 0 – internal PRBS signal, 1 – 3GGP test model | 0 |
|  | [9:2] | RW | TEST_EXAC_EN_MASK | 8 | Bit mask to enable generating test signal for each eAxC in a parallel. LSB – eAxC_0, MSB – eAxC_7 | 0x1 |
|  | [11:10] | RW | TEST_INT_MOD_TYPE | 2 | Modulation type for internal PRBS generator: 0 – QPSK, 1 – 16QAM, 2 – 64QAM, 3 – 256QAM | 0 |
|  | [14:12] | RW | TEST_TM_INDX | 3 | 3GPP test model index for test signal generating | 0 |
| 0x11 | [31:0] | RW | TEST_TM11_DDR_ADDR | 32 | 3GPP TM1.1 starting address in DDR | n/s |
| 0x12 | [31:0] | RW | TEST_TM12_DDR_ADDR | 32 | 3GPP TM1 starting address in DDR | n/s |
| 0x13 | [31:0] | RW | TEST_TM2_DDR_ADDR | 32 | 3GPP TM2 starting address in DDR | n/s |
| 0x14 | [31:0] | RW | TEST_TM2A_DDR_ADDR | 32 | 3GPP TM2a starting address in DDR | n/s |
| 0x15 | [31:0] | RW | TEST_TM31_DDR_ADDR | 32 | 3GPP TM3.1 starting address in DDR | n/s |
| 0x16 | [31:0] | RW | TEST_TM31A_DDR_ADDR | 32 | 3GPP TM3.1A starting address in DDR | n/s |
| 0x17 | [31:0] | RW | TEST_TM32_DDR_ADDR | 32 | 3GPP TM3.2 starting address in DDR | n/s |
| 0x18 | [31:0] | RW | TEST_TM33_DDR_ADDR | 32 | 3GPP TM3.3 starting address in DDR | n/s |
| 0x19 | [31:0] | RW | TEST_TM_LENGHT | 32 | Length of generated test signal for one frame | n/s |
| Beam Cache Configuration |  |  |  |  |  |  |
| 0x20 | [0] | RW | BC_OP_TYPE | 1 | Operation type: 0 – Read, 1 - Wrire Beam Cache table item | 0 |
|  | [2:1] | RW | BC_TABLE_SEL | 2 | Select a destination/source for getting access to Beam Cache: 0 – DL & UL, 1 – DL only, 2 – UL only, 3 – PRACH Note: Reading for 0x0 value returns content of DL Beam Cache Table | 0 |
|  | [11:3] | RW | BC_TABLE_ ADDR | 9 | Address of Beam Cache Table item address | 0 |
| 0x21 | [31:0] | RW | BC_TABLE_VAL[31:0] | 32 | Beam Cache Table item value [31:0] bits |  |
| 0x22 | [31:0] | RW | BC_TABLE_VAL[63:32] | 32 | Beam Cache Table item value [63:32] bits |  |
| 0x23 | [31:0] | RW | BC_TABLE_VAL[95:64] | 32 | Beam Cache Table item value [95:64] bits |  |
| 0x24 | [31:0] | RW | BC_TABLE_VAL[127:96] | 32 | Beam Cache Table item value [127:96] bits |  |
| 0x25 | [31:0] | RW | BC_TABLE_VAL[159:128] | 32 | Beam Cache Table item value [159:128] bits |  |
| 0x26 | [31:0] | RW | BC_TABLE_VAL[191:160] | 32 | Beam Cache Table item value [191:160] bits |  |
| Statistic Counters |  |  |  |  |  |  |
| 0x80 | [31:0] | RO | DL_FRAMES_CNT | 32 | Number of frames in downlink |  |
| 0x81 | [31:0] | RO | UL_FRAMES_CNT | 32 | Number of frames in uplink |  |
| 0xA0 | [31:0] | RO | TM_FRAMES_CNT | 32 | Number of generated TM frames |  |
| 0xA1 | [31:0] | RO | TM_EXC_EMPTY_FIFO_RD | 32 | Number of frames with the exceptional situation of TM data request when |  |
| 0xA2 | [31:0] | RO | TM_EXC_OVER_LEN_RD | 32 | Number of frames with the exceptional situation of requesting data over TM_LENGTH |  |
| 0xA3 | [31:0] | RO | TM_EXC_DMA_RD | 32 | Number of frames with the exceptional situation of non-full reading TM_LENGTH data with frame period by DMA |  |
| 0xA4 | [31:0] | RO | TM_EXC_TMGEN_RD | 32 | Number of frames with the exceptional situation of non-full reading TM_LENGTH data with frame period by DMA |  |
| 0xB0 | [31:0] | RO | DSPU_NEXT_FRAME_CNTR | 32 | Number of frames in DL DSPU |  |
| 0xB1 | [31:0] | RO | DSPU_NEXT_SLOT_CNTR | 32 | Number of slots in DL DSPU |  |
| 0xB2 | [31:0] | RO | DSPU_0_EMPTY_SLOT_CNTR | 32 | Number of empty slots for DSPU 0 when BID params didn’t come |  |
| 0xB3 | [31:0] | RO | DSPU_0_BID_PARAMS_CNTR | 32 | Number of BID params transactions which came to DSPU 0 |  |
| 0xB4 | [31:0] | RO | DSPU_0_INPUT_PACKET_CNTR | 32 | Number of data packets which came to DSPU 0 |  |
| 0xB5 | [31:0] | RO | DSPU_0_OUTPUT_PACKET_CNTR | 32 | Number of data packets which came from CDC unit in DSPU 0 |  |
| 0xB6 | [31:0] | RO | DSPU_0_END_OFDM_SYMB_MISM | 32 | Number of incorrect OFDM symbols in DSPU 0 |  |
| 0xB8 | [31:0] | RO | DSPU_1_EMPTY_SLOT_CNTR | 32 | Number of empty slots for DSPU 1 when BID params didn’t come |  |
| 0xB9 | [31:0] | RO | DSPU_1_BID_PARAMS_CNTR | 32 | Number of BID params transactions which came to DSPU 1 |  |
| 0xBA | [31:0] | RO | DSPU_1_INPUT_PACKET_CNTR | 32 | Number of data packets which came to DSPU 1 |  |
| 0xBB | [31:0] | RO | DSPU_1_OUTPUT_PACKET_CNTR | 32 | Number of data packets which came from CDC unit in DSPU 1 |  |
| 0xBC | [31:0] | RO | DSPU_1_END_OFDM_SYMB_MISM | 32 | Number of incorrect OFDM symbols in DSPU 1 |  |
| 0xC0 | [31:0] | RO | DSPU_2_EMPTY_SLOT_CNTR | 32 | Number of empty slots for DSPU 2 when BID params didn’t come |  |
| 0xC1 | [31:0] | RO | DSPU_2_BID_PARAMS_CNTR | 32 | Number of BID params transactions which came to DSPU 2 |  |
| 0xC2 | [31:0] | RO | DSPU_2_INPUT_PACKET_CNTR | 32 | Number of data packets which came to DSPU 2 |  |
| 0xC3 | [31:0] | RO | DSPU_2_OUTPUT_PACKET_CNTR | 32 | Number of data packets which came from CDC unit in DSPU 0 |  |
| 0xC4 | [31:0] | RO | DSPU_2_END_OFDM_SYMB_MISM | 32 | Number of incorrect OFDM symbols in DSPU 2 |  |
| 0xC8 | [31:0] | RO | DSPU_3_EMPTY_SLOT_CNTR | 32 | Number of empty slots for DSPU 3 when BID params didn’t come |  |
| 0xC9 | [31:0] | RO | DSPU_3_BID_PARAMS_CNTR | 32 | Number of BID params transactions which came to DSPU 3 |  |
| 0xCA | [31:0] | RO | DSPU_3_INPUT_PACKET_CNTR | 32 | Number of data packets which came to DSPU 3 |  |
| 0xCB | [31:0] | RO | DSPU_3_OUTPUT_PACKET_CNTR | 32 | Number of data packets which came from CDC unit in DSPU 3 |  |
| 0xCC | [31:0] | RO | DSPU_3_END_OFDM_SYMB_MISM | 32 | Number of incorrect OFDM symbols in DSPU 3 |  |
| 0xD0 | [31:0] | RO | DSPU_4_EMPTY_SLOT_CNTR | 32 | Number of empty slots for DSPU 4 when BID params didn’t come |  |
| 0xD1 | [31:0] | RO | DSPU_4_BID_PARAMS_CNTR | 32 | Number of BID params transactions which came to DSPU 4 |  |
| 0xD2 | [31:0] | RO | DSPU_4_INPUT_PACKET_CNTR | 32 | Number of data packets which came to DSPU 4 |  |
| 0xD3 | [31:0] | RO | DSPU_4_OUTPUT_PACKET_CNTR | 32 | Number of data packets which came from CDC unit in DSPU 4 |  |
| 0xD4 | [31:0] | RO | DSPU_4_END_OFDM_SYMB_MISM | 32 | Number of incorrect OFDM symbols in DSPU 4 |  |
| 0xD8 | [31:0] | RO | DSPU_5_EMPTY_SLOT_CNTR | 32 | Number of empty slots for DSPU 5 when BID params didn’t come |  |
| 0xD9 | [31:0] | RO | DSPU_5_BID_PARAMS_CNTR | 32 | Number of BID params transactions which came to DSPU 5 |  |
| 0xDA | [31:0] | RO | DSPU_5_INPUT_PACKET_CNTR | 32 | Number of data packets which came to DSPU 5 |  |
| 0xDB | [31:0] | RO | DSPU_5_OUTPUT_PACKET_CNTR | 32 | Number of data packets which came from CDC unit in DSPU 5 |  |
| 0xDC | [31:0] | RO | DSPU_5_END_OFDM_SYMB_MISM | 32 | Number of incorrect OFDM symbols in DSPU 5 |  |
| 0xE0 | [31:0] | RO | DSPU_6_EMPTY_SLOT_CNTR | 32 | Number of empty slots for DSPU 6 when BID params didn’t come |  |
| 0xE1 | [31:0] | RO | DSPU_6_BID_PARAMS_CNTR | 32 | Number of BID params transactions which came to DSPU 6 |  |
| 0xE2 | [31:0] | RO | DSPU_6_INPUT_PACKET_CNTR | 32 | Number of data packets which came to DSPU 6 |  |
| 0xE3 | [31:0] | RO | DSPU_6_OUTPUT_PACKET_CNTR | 32 | Number of data packets which came from CDC unit in DSPU 6 |  |
| 0xE4 | [31:0] | RO | DSPU_6_END_OFDM_SYMB_MISM | 32 | Number of incorrect OFDM symbols in DSPU 6 |  |
| 0xE8 | [31:0] | RO | DSPU_7_EMPTY_SLOT_CNTR | 32 | Number of empty slots for DSPU 7 when BID params didn’t come |  |
| 0xE9 | [31:0] | RO | DSPU_7_BID_PARAMS_CNTR | 32 | Number of BID params transactions which came to DSPU 7 |  |
| 0xEA | [31:0] | RO | DSPU_7_INPUT_PACKET_CNTR | 32 | Number of data packets which came to DSPU 7 |  |
| 0xEB | [31:0] | RO | DSPU_7_OUTPUT_PACKET_CNTR | 32 | Number of data packets which came from CDC unit in DSPU 7 |  |
| 0xEC | [31:0] | RO | DSPU_7_END_OFDM_SYMB_MISM | 32 | Number of incorrect OFDM symbols in DSPU 7 |  |
| 0xF0 | [31:0] | RO | SSB_EMPTY_SLOT_CNTR | 32 | Number of empty slots for SSB DSPU when BID params didn’t come |  |
| 0xF1 | [31:0] | RO | SSB_BID_PARAMS_CNTR | 32 | Number of BID params transactions which came to SSB DSPU |  |
| 0xF2 | [31:0] | RO | SSB_INPUT_PACKET_CNTR | 32 | Number of data packets which came to SSB DSPU |  |
| 0xF3 | [31:0] | RO | SSB_OUTPUT_PACKET_CNTR | 32 | Number of data packets which came from CDC unit in SSB DSPU |  |
| 0xF4 | [31:0] | RO | SSB_END_OFDM_SYMB_MISM | 32 | Number of incorrect OFDM symbols in SSB DSPU |  |
| 0x100 | [31:0] | RO | SFU_NEXT_FRAME_CNT_0 | 32 | Number of input frames to UL SFU from CH0 |  |
| 0x101 | [31:0] | RO | SFU_NEXT_FRAME_CNT_1 | 32 | Number of input frames to UL SFU from CH1 |  |
| 0x102 | [31:0] | RO | OFPU_NEXT_FRAME_CNT | 32 | Number of frames in UL OFPU |  |
| 0x103 | [31:0] | RO | OFPU_NEXT_SLOT_CNT | 32 | Number of slots in UL OFPU |  |
| 0x104 | [31:0] | RO | SFU_NON_EMPTY_FIFO | 32 | Number of frames where start frame signal was received for non-empty FIFOs |  |
| 0x105 | [31:0] | RO | SFU_CH_LARGE_SHIFT | 32 | Number of clock cycles where the shift between CH0 and CH1 is higher then defined limit (TBD) |  |
| 0x106 | [31:0] | RO | SFU_EMPTY_FIFO_RD | 32 | Number of data request from Input FIFO by MACx256 |  |
| 0x108 | [31:0] | RO | OFPU_0_EMPTY_SLOT_CNT | 32 | Number of empty slots for OFPU 0 when BID params didn’t come |  |
| 0x109 | [31:0] | RO | OFPU_0_BID_PARAMS_CNT | 32 | Number of BID params transactions which came to OFPU 0 |  |
| 0x10A | [31:0] | RO | OFPU_0_IN_REQ_CNT | 32 | Number of data requests which came to OFPU 0 |  |
| 0x10B | [31:0] | RO | OFPU_0_OUT_AXIS_CNT | 32 | Number of data AXIS transactions to ORAN framer from OFPU 0 |  |
| 0x10C | [31:0] | RO | OFPU_0_IN_WRITE_EXC | 32 | Number of OFDM symbols with no input to OFPU 0 from MACx256 |  |
| 0x110 | [31:0] | RO | OFPU_1_EMPTY_SLOT_CNT | 32 | Number of empty slots for OFPU 0 when BID params didn’t come |  |
| 0x111 | [31:0] | RO | OFPU_1_BID_PARAMS_CNT | 32 | Number of BID params transactions which came to OFPU 0 |  |
| 0x112 | [31:0] | RO | OFPU_1_IN_REQ_CNT | 32 | Number of data requests which came to OFPU 0 |  |
| 0x113 | [31:0] | RO | OFPU_1_OUT_AXIS_CNT | 32 | Number of data AXIS transactions to ORAN framer from OFPU 0 |  |
| 0x114 | [31:0] | RO | OFPU_1_IN_WRITE_EXC | 32 | Number of OFDM symbols with no input to OFPU 0 from MACx256 |  |
| 0x118 | [31:0] | RO | OFPU_2_EMPTY_SLOT_CNT | 32 | Number of empty slots for OFPU 2 when BID params didn’t come |  |
| 0x119 | [31:0] | RO | OFPU_2_BID_PARAMS_CNT | 32 | Number of BID params transactions which came to OFPU 2 |  |
| 0x11A | [31:0] | RO | OFPU_2_IN_REQ_CNT | 32 | Number of data requests which came to OFPU 2 |  |
| 0x11B | [31:0] | RO | OFPU_2_OUT_AXIS_CNT | 32 | Number of data AXIS transactions to ORAN framer from OFPU 2 |  |
| 0x11C | [31:0] | RO | OFPU_2_IN_WRITE_EXC | 32 | Number of OFDM symbols with no input to OFPU 2 from MACx256 |  |
| 0x120 | [31:0] | RO | OFPU_3_EMPTY_SLOT_CNT | 32 | Number of empty slots for OFPU 3 when BID params didn’t come |  |
| 0x121 | [31:0] | RO | OFPU_3_BID_PARAMS_CNT | 32 | Number of BID params transactions which came to OFPU 3 |  |
| 0x122 | [31:0] | RO | OFPU_3_IN_REQ_CNT | 32 | Number of data requests which came to OFPU 3 |  |
| 0x123 | [31:0] | RO | OFPU_3_OUT_AXIS_CNT | 32 | Number of data AXIS transactions to ORAN framer from OFPU 3 |  |
| 0x124 | [31:0] | RO | OFPU_3_IN_WRITE_EXC | 32 | Number of OFDM symbols with no input to OFPU 3 from MACx256 |  |
| 0x128 | [31:0] | RO | OFPU_4_EMPTY_SLOT_CNT | 32 | Number of empty slots for OFPU 4 when BID params didn’t come |  |
| 0x129 | [31:0] | RO | OFPU_4_BID_PARAMS_CNT | 32 | Number of BID params transactions which came to OFPU 4 |  |
| 0x12A | [31:0] | RO | OFPU_4_IN_REQ_CNT | 32 | Number of data requests which came to OFPU 4 |  |
| 0x12B | [31:0] | RO | OFPU_4_OUT_AXIS_CNT | 32 | Number of data AXIS transactions to ORAN framer from OFPU 4 |  |
| 0x12C | [31:0] | RO | OFPU_4_IN_WRITE_EXC | 32 | Number of OFDM symbols with no input to OFPU 4 from MACx256 |  |
| 0x130 | [31:0] | RO | OFPU_5_EMPTY_SLOT_CNT | 32 | Number of empty slots for OFPU 5 when BID params didn’t come |  |
| 0x131 | [31:0] | RO | OFPU_5_BID_PARAMS_CNT | 32 | Number of BID params transactions which came to OFPU 5 |  |
| 0x132 | [31:0] | RO | OFPU_5_IN_REQ_CNT | 32 | Number of data requests which came to OFPU 5 |  |
| 0x133 | [31:0] | RO | OFPU_5_OUT_AXIS_CNT | 32 | Number of data AXIS transactions to ORAN framer from OFPU 5 |  |
| 0x134 | [31:0] | RO | OFPU_5_IN_WRITE_EXC | 32 | Number of OFDM symbols with no input to OFPU 5 from MACx256 |  |
| 0x138 | [31:0] | RO | OFPU_6_EMPTY_SLOT_CNT | 32 | Number of empty slots for OFPU 6 when BID params didn’t come |  |
| 0x139 | [31:0] | RO | OFPU_6_BID_PARAMS_CNT | 32 | Number of BID params transactions which came to OFPU 6 |  |
| 0x13A | [31:0] | RO | OFPU_6_IN_REQ_CNT | 32 | Number of data requests which came to OFPU 6 |  |
| 0x13B | [31:0] | RO | OFPU_6_OUT_AXIS_CNT | 32 | Number of data AXIS transactions to ORAN framer from OFPU 6 |  |
| 0x13C | [31:0] | RO | OFPU_6_IN_WRITE_EXC | 32 | Number of OFDM symbols with no input to OFPU 6 from MACx256 |  |
| 0x140 | [31:0] | RO | OFPU_7_EMPTY_SLOT_CNT | 32 | Number of empty slots for OFPU 7 when BID params didn’t come |  |
| 0x141 | [31:0] | RO | OFPU_7_BID_PARAMS_CNT | 32 | Number of BID params transactions which came to OFPU 7 |  |
| 0x142 | [31:0] | RO | OFPU_7_IN_REQ_CNT | 32 | Number of data requests which came to OFPU 7 |  |
| 0x143 | [31:0] | RO | OFPU_7_OUT_AXIS_CNT | 32 | Number of data AXIS transactions to ORAN framer from OFPU 7 |  |
| 0x144 | [31:0] | RO | OFPU_7_IN_WRITE_EXC | 32 | Number of OFDM symbols with no input to OFPU 7 from MACx256 |  |
| 0x14 | [31:0] | RO | DL_PARS_CH_ID_ERR | 32 | Error channel ID for DL stream parser | 0 |
| 0x15 | [31:0] | RO | UL_PARS_CH_ID_ERR | 32 | Error channel ID for UL stream parser | 0 |