# MRU Framer Register Map – FRAMv0.3.0

| Register | Bits | Type | Name | Bit Num | Description | Value |
| --- | --- | --- | --- | --- | --- | --- |
| 0x0 | [31:0] | RO | IP_VERSION | 32 | IP core version in the following X.Y.Z format <Version_Num[7:0], Revision_Num[7:0], Patch_Num[7:0]> | n/s |
| 0x1 | [0] | RW | KILL | 1 | A synchronous reset of total IP block. The high value corresponds to the reset state | 0 |
|  | [1] | RWP | STAT_KILL | 1 | A synchronous reset of total statistic counters | 0 |
| 0x2 | [0] | RW | NEXT_FRAME_IRQ_EN | 1 | Enable interrupt of next frame event | 0 |
|  | [1] | RW | NEXT_FRAME_IRQ_RST | 1 | Reset next frame interrupt | 0 |
|  | [2] | RW | AXI4L_BRESP_IRQ_EN | 1 | AXI4-lite BRESP interrupt enable | 0 |
|  | [3] | RWP | AXI4L_BRESP_IRQ_RST | 1 | Reset AXI4-lite BRESP interrupt | 0 |
| 0x3 | [0] | RW | INT_RTC_EN | 1 | Enable internal RTC usage for next frame generating | 0 |
|  | [25:1] | RW | INT_RTC_PERIOD | 25 | Period of internal RTC calculated in fixed point format as round(Tflp_ns×2^20), where Tflp_ns – period float value in nanosec. |  |
|  | [26] | RW | FRAME_SRC_TYPE | 1 | Source of framer period: 0 – Internal clock counter (Mode 0), 1 – RTC (internal or external) (Mode 1) | 1 |
|  | [27] | RW | FRAME_PPS_SYNC_EN | 1 | Aligning next frame pulse with input PPS signal: 0 – Enable, 1 – Disable | 1 |
|  | [28] | RW | PPS_ONCE_RESYNC | 1 | PPS Resync mode: 0 – continuous, 1 – once only | 0 |
| 0x4 | [31:0] | RW | FRAME_PERIOD_NS | 32 | One frame period in nanosec for Mode 1 | 1e9 |
| 0x5 | [23:0] | RW | DL_ORAN_NF_SHIFT | 24 | Shift in 3x clock cycles next_frame signal for ORAN Framer (UL) | 0 |
| 0x6 | [23:0] | RW | UL_ORAN_NF_SHIFT | 24 | Shift in 3x clock cycles next_frame signal for ORAN Framer (UL) | 0 |
| 0x7 | [23:0] | RW | DL_DFE_NF_SHIFT | 24 | Shift in 3x clock cycles next_frame signal for DFE DL Subsystem Framer | 0 |
| 0x8 | [23:0] | RW | UL_DFE_NF_SHIFT | 24 | Shift in 3x clock cycles next_frame signal for DFE UL Subsystem Framer | 0 |
| 0x9 | [31:0] | RW | SLOT_PERIOD_NS | 32 | One slot period in nanosec | 0.5e6 |
| 0xA | [23:0] | RW | DL_NS _SHIFT | 24 | Shift in 3x clock cycles next_slot signal for DL puropses | 0 |
| 0xB | [23:0] | RW | UL_NS _SHIFT | 24 | Shift in 3x clock cycles next_slot signal for UL puropses | 0 |
| 0xC | [31:0] | RW | FRAME_PERIOD_CLKS | 31 | Frame period in clock cycles for Mode 0 | 368-6400 |
| 0xD | [31:0] | RW | SLOT_PERIOD_CLKS | 31 | Slot period in clock cycles for Mode 0 | 184-320 |
| 0x10 | [31:0] | RO | FRAME_CNT | 31 | Number of generated frame pulses | n/s |
| 0x11 | [31:0] | RO | SLOT_CNT | 31 | Number of generated slots pulses | n/s |
| 0x12 | [31:0] | RO | EXC_DBL_NEXT_FRAME | 31 | Number of exceptions with double next_frame pulses within one frame peiod | n/s |
| 0x20 | [31:0] | RO | ESTIM_FRAME_PERIOD | 31 | The last estimation of a frame period in PHY clock cycles | n/s |
| 0x21 | [31:0] | RO | ESTIM_SLOT_PERIOD | 31 | The last estimation of a slot period in PHY clock cycles | n/s |
| 0x14 | [31:0] | RO | DL_PARS_CH_ID_ERR | 32 | Error channel ID for DL stream parser | 0 |
| 0x15 | [31:0] | RO | UL_PARS_CH_ID_ERR | 32 | Error channel ID for UL stream parser | 0 |