Compilation steps for I2C Interface

Step1:

Copy the image to the SD card and insert into board.

Step2:

Enter to Board and change permissions for i2c device using below command:

$chmod 777 /dev/i2c-1

Step3:

Execute the test suite using below command:

$Diagnostic_Menu

Step4:

Enter option-1 to select SDF FPGA PS board.

Step5:

Enter option-5 to select I2C interface.

It displayed the data that has been read from the EEPROM.

Step6:

Select option 6 to Quit.

In BOOT COMMAND Prompt:

Give the following commands:

Command 1:

$i2c dev 1

Setting i2c bus to 1

Command 2:

$i2c md 0x50 0 0x100

0x50 – Address of EEPROM

0 – Starting address of data

0x100 – Ending address of data