Compilation steps for User Application

Step1:

Create an application by using the command petalinux-create -t apps --name <appname> --enable in the project path.

For example: 1. cd /home/airbus/Evaluation/kr260/ ---> Go to project folder

2. petalinux-create -t apps --name testapp --enable

Step2:

Go to the path /home/airbus/Evaluation/kr260/project-spec/meta-user/recipes-apps/files/<filename>.c and replace with application code in <filename>.c file

Step3:

Need to verify that the created file is included in rootfs by using the below command:

Petalinux-config -c rootfs ----> apps ---> <filename>

Step4:

Build the petalinux application to verify any issue in application by using the command

petalinux-build -c <appname>

Step5:

Build the petalinux for overall project by using command

petalinux-build

Compilation steps for custom modules

Step1:

Create a kernel module by using the command

petalinux-create -t modules --name <modulename> --enable in the project path.

Note – Filename should be in lowercase

For example: 1. cd /home/airbus/Evaluation/kr260/ ---->Go to project folder

2. petalinux-create -t modules --name  testmodule --enable

Step2:

Go to the path /home/airbus/Evaluation/kr260/project-spec/meta-user/recipes-modules/files/<filename>.c and replace with driver code in <filename>.c file

Step3:

Need to verify that the created file is included in rootfs by using the below command:

Petalinux-config -c rootfs ----> modules ---> <filename>

Step4:

Build the petalinux for driver by using the command

petalinux-build -c <modulename>

Step5:

Build the petalinux for overall project by using command

petalinux-build

Manual partition in SD card

Step1:

Create Partition by using command fdisk <device>(for example-/dev/sda)

Command (m for help): n

Select (default p): p

Partition number (1-4, default 1): 1

First sector (2048-62333951, default 2048):

Last sector, +sectors or +size{K,M,G,T,P} (2048-62333951, default 62333951): +1G

Do you want to remove the signature? [Y]es/[N]o: y

Command (m for help): n

Select (default p): p

Partition number (1-4, default 1): 2

First sector (2048-62333951, default 2048):

Last sector, +sectors or +size{K,M,G,T,P} (2048-62333951, default 62333951):

Command (m for help): w

- Command used to format the fat file system

$ sudo mkfs.vfat <device1> (For example: sudo mkfs.vfat /dev/sda1)

- Command used to format the ext file system

$ sudo mkfs.ext4 <device2> (For example: sudo mkfs.ext4 /dev/sda2)

- Copy the below images into vfat file system by using following commands:

$ sudo mount <device1> /mnt (For example: sudo mount /dev/sda1 /mnt)

$ cp -r BOOT.BIN boot.scr image.ub /mnt

$ sudo umount /mnt

- Copy the driver into ext4 file system by using the following commands:

$ sudo mount <device2> /mnt (For example: sudo mount /dev/sda2 /mnt)

$ cp -r testmodule.ko /mnt

$ sudo umount /mnt

Creating wic image

- Create wic image with the command petalinux-package –wic –images-dir images/linux/ --bootfiles “rootfs.cpio.gz.u-boot,boot.scr,Image,system-zynqmp-sck-kr-g-revB.dtb,BOOT.BIN”

Note:  Either create wic image or manual partition to copy the images into SD card.

Testing driver and application in Board

Step 1:

Insert the SD card into the board and power on.

Step2:

Go to the folder where file copied in the SD card and Insert the driver by using following command

sudo insmod <filename>.ko

For example:

Sudo insmode testmodule.ko

Step 3:

Create a device file by using the command

sudo mknod /dev/<device file name> c <major number> <minor number>

For example:

sudo mknod /dev/ModuleDiagnostic c 501 0

Step 4:

Change the permissions for device file by using the below command

chmod 777 /dev/<device file name>

For example:

chmod 777 /dev/ModuleDiagnostic

Step 5:

Execute application as below:

<application name>

For example:

testapp