Loading BIN file in Quad SPI

Step1:

Refer Compilation steps Document upto BOOT.BIN file got generated.

Step2:

Copy BOOT.BIN file into SD card as below:

cp -r BOOT.BIN /dev/<device>

For example: cp -r BOOT.BIN /dev/sda

Step3:

Turn on the board and check the status of BOOT.BIN which is loading by giving below command:

xmutil bootfw_status

Step4:

For updating the bin file, use the below command:

xmutil bootfw_update -i BOOT.BIN

Step5:

Check the status of boot file by using the below command:

xmutil bootfw_status

Step6:

To boot the latest image boot file, use below command:

xmutil bootfw_update -v

Step7:

To check from which image it is booting from, use the command

xmutil bootfw_status

Note: Flash the BIN file only once.