Petalinux Compilation Steps

Petalinux building:

Install petalinux to our machine for that we need petalinux_installer.run . Download the file w.r.t version.

- First change the permission for .run file to 755 using (chmod 755 ./*.run)

- Create a directory where sdk petalinux should be installed.(eg: mkdir sample/test/)

- Now give command ./petalinux-v2021.2-final-installer.run --dir  test/

Compilation:

- Give command source test/settings.sh

- Create a petalinux project with the file having extension .bsp which is located in /home/airbus/april9/

- Give command petalinux-create -t project -s xilinx-kr260-starterkit-v2022.1-05140151.bsp

- Go to xilinx-kr260-starterkit-2022.1 directory.

- We have some config files like .xsa,.bit files have to take from fpga

- Give command petalinux-config --get-hw-description /home/airbus/Downloads/<filename>.xsa

- Now to build it give command petalinux-build

- Create a image file which will have BOOT.BIN, image.ub, boot.scr for that

- Give command petalinux-package --boot --format BIN --fsbl images/linux/zynqmp_fsbl.elf --u-boot images/linux/u-boot.elf --pmufw images/linux/pmufw.elf --fpga /home/airbus/Downloads/<filename>.bit –force

- Create  wic image with command petalinux-package –wic –images-dir images/linux/ --bootfiles “rootfs.cpio.gz.u-boot,boot.scr,Image,system.dtb,system-zynqmp-sck-kr-g-revB.dtb,BOOT.BIN”

- Copy the wic file into SD card by uding command sudo dd if=<File Name.wic> of=/dev/<Device Name> status=progress conv=fsync