SDK STEPS:

- Make sure these packages are installed.

- $ sudo apt-get update

- $ # Install packages required for builds

- $ sudo apt-get -f -y install \

git build-essential diffstat texinfo gawk chrpath socat doxygen \

dos2unix python3 bison flex libssl-dev u-boot-tools mono-devel \

mono-complete curl python3-distutils repo pseudo python3-sphinx \

g++-multilib libc6-dev-i386 jq git-lfs pigz zstd liblz4-tool \

cpio file zstd lz4

- Build Steps

- $ git clone https://git.ti.com/git/arago-project/oe-layersetup.git tisdk

- $ cd tisdk

- $ ./oe-layertool-setup.sh -f configs/processor-sdk/<oeconfig-file>

- $ cd build

- $ . conf/setenv

- $ MACHINE=<machine> bitbake -k tisdk-default-image

- If you are facing any issue while cloning give command

  - sudo route add default gw 192.168.1.1 <interface name>

  - interface name example eth0

- If you are facing locale.Error: unsupported locale setting error give this command

  - export LC_ALL="en_US.UTF-8"

  - export LC_CTYPE="en_US.UTF-8"

  - export LC_NUMERIC="en_US.UTF-8"

  - export LANG=en_US.UTF-8

  - export LANGUAGE=en_US.UTF-8

  - ulimit -n 4096

- To build image run the following command:

  - MACHINE=<machine> bitbake -k <target>

  - machine= am62xx-evm, target = tisdk-base-image or tisdk-default-image

- To change source code that is fetched from git and to compile recipes go to following path change and give the below command.

  - Path: tisdk/build/arago-tmp-default-glibc/work/am62xx_evm-oe-linux/<Recipename>/git/

  - Add changes in source file and run:

    - MACHINE=<machine> bitbake <recipename> --force -c compile

    - MACHINE=<machine> bitbake <recipename>

    - MACHINE=<machine> bitbake -k <target>

- To clean recipe and build again run following command.

    - MACHINE=<machine> bitbake <target> -c cleanall

    - MACHINE=<machine> bitbake <recipename>

BOOTING STEPS

SD CARD BOOT:

- Download the default bootable SD card image (WIC file) available on the release page as tisdk-default-image-<machine>.wic.xz. The Processor SDK Linux AM62x installer also contains other WIC images that can be flashed to the SD card similarly. Assuming the SDK is installed on a Linux machine at <PSDK_PATH>, you can find other available WIC images at <PSDK_PATH>/filesystem/

- Download and install the balenaEtcher tool

- Flash the WIC image to the SD card:

  - insert a micro SD card into the USB SD card reader and start Etcher.Choose the default WIC image to be flashed, choose the USB SD card reader as the target, and then click “Flash”. Etcher will decompress the image and write it to the SD card, as shown below:

EMMC boot through Sd card:

1. In target where evm bring up using sd card copy all required images using scp

- tiboot3.bin

- tispl.bin

- u-boot-<EVM name>.img

2. after give command : echo 0 > /sys/block/mmcblk0boot0/force_ro

3. write the binaries to eMMC boot0

- # dd if=tiboot3.bin of=/dev/mmcblk0boot0 seek=0

- # dd if=tispl.bin of=/dev/mmcblk0boot0 seek=1024

- # dd if=u-boot.img of=/dev/mmcblk0boot0 seek=5120

4.Disk partition.

  - fdisk /dev/mmcblk0

  - Enter “p” to display current partitions

  - Enter “n” to create a new partition

  - Enter “w” to write the partition table to disk and exit

  - mkfs.ext4 /dev/mmcblk0p1

5.Copy the Linux image from the host to the EVM. For example, the default image for AM62x:

  - mkdir -p /mnt/temp

  - # mount -t ext4 /dev/mmcblk0p1 /mnt/temp

  - # cd /mnt/temp

  - # tar xf <Linux image file copied from the host>

  - # cd ..

  - # umount temp

6. Reboot the board without changing any switch and stop at u-boot and run following commands.

èmmc partconf 0 1 1 1

èmmc bootbus 0 2 0 0

7.Now poweroff board change switches to emmc mode and power on and stop at u-boot and enter following command:

èsetenv mmcdev 0

èsetenv bootpart 0

èboot

DFU BOOT:

- Here we need two set of images

  - One is configure with dfuusb and other is normal images.

  - By default without changing any config build one image that is one set copy and save the required images in folder let us call emmc_image.

  - In tisdk/sources/meta-ti/meta-ti-bsp/conf/machines/am62xx-evm-k3r5.conf set machine name to “am62x_evm_r5_usbdfu_defconfig” and build image this is other set let us call dfu_image.

- First set switches to usb-peripheral switch mode now give following commands by entering into dfu_images folder in host machine:

  - sudo dfu-util -R -a bootloader -D tiboot3.bin

  - sudo dfu-util -R -a tispl.bin -D tispl.bin

  - sudo dfu-util -R -a u-boot.img -D u-boot.img

- Now when you loaded u-boot.img go to board terminal and stop at u-boot then enter the following commands in board terminal:

  - setenv dfu_alt_info ${dfu_alt_info_emmc}

  - dfu 0 mmc 0

- Now turn off the board and change boot mode switch to emmc boot.

- Now in emmc_images folder add run .sh file to generate .ext4 file after give the following commands in host machine:

  - sudo dfu-util -a tiboot3.bin.raw -D <your_tiboot3.bin>

  - sudo dfu-util -a tispl.bin.raw -D <your_tispl.bin>

  - sudo dfu-util -a u-boot.img.raw -D <your_u-boot.img>

  - sudo dfu-util -a rootfs -D <your_rootfs.ext4>

- after successfully images were loaded give the ROM access to the boot partition by giving following commands in board terminal:

=> mmc partconf 0 1 1 1
        => mmc bootbus 0 2 0 0

- To boot kernel from eMMC, use the following commands in board terminal during u-boot:

  - setenv mmcdev 0

  - setenv bootpart 0

  - boot

Reduce the WKUP_CLKOUT0 clock output to 10 MHz:

Steps:

devmem2 0x682010 w 0x68EF3490

devmem2 0x682014 w 0xD172BC5A

devmem2 0x006820a4 w 0x00008063

devmem2 0x109008 w 0x68EF3490

devmem2 0x10900c w 0xD172BC5A

devmem2 0x43008020 w 4b100